Document No:                                                                                     Revision:

Issue No:

Date of Issue:

PRALAY QT/AT DOCUMENT

SL.NO –

Prepared By                                                                      Reviewed By

Dy.Director,R&QA

Approved By

AMENDMENT RECORD

| Sl. No | Description | Amendment No. | Authority | Date | Affected Drawings/Sheets in QT/AT Document |
| --- | --- | --- | --- | --- | --- |

Note:  Holder of the document should fill table as and when Amendment / Addendum is received.

Contents

LIST OF ABBREVIATIONS	4

1.0     SCOPE OF THE DOCUMENT	6

2.0     LIST OF DRAWINGS	6

3.0 LIST OF STANDARDS	7

4.0 APPLICABLE DOCUMENTS	7

5.0 INTRODUCTION	8

6.0 DESCRIPTION	8

7.0 SPECIFICATIONS	11

7.1 Specifications of OBC Card:	11

7.2 Specifications of MIU Card:	11

7.3 Specifications of INS Card:	12

7.4 Specifications of GPS+GLONASS/IRNSS RECEIVER	12

7.5  Qualification Tests Specifications	13

7.6   Acceptance Tests Specifications	16

7.7  EMI/EMC as per MIL – STD  - 461 - E Specifications	17

8.0 LIST OF DELIVERABLES	20

9.0 READINESS OF QT/AT	21

9.1 READINESS OF TEST EQUIPMENT/FACILITIES	21

9.2 Safety Measures	22

9.3 Availability of logbook	22

10 SEQUENCE OF TESTS	23

10.1 Qualification Tests	23

10.2 Acceptance Tests	23

11. TEST PROCEDURES	24

11.1 Functional Test Procedure	24

Power On Self Test for OBC Resources:-	26

11.2  Test Set – Up	27

11.2.1  Test  Setup  Procedure	27

11.3 Initial And Final Performance Checks	29

11.4 Qualification Tests	29

11.5 Acceptance Tests	29

11.6 Burn-In Test	30

11.7 Vibration Test	31

11.8  Thermal Cycling Test	32

11.9  Damp Heat	34

11.10  Shock Test	34

11.11 Bump Test	35

11.12 Tropical Exposure	36

11.13 Acceleration	37

12  FORMATS	39

12.1 Unit Configuration	39

12.2 Quality Assessment Report	40

12.3 ACCEPTANCE TEST REPORT FORMATS	41

ACCEPTANCE TEST LEAF  - 1	41

ACCEPTANCE TEST LEAF  - 2	42

ACCEPTANCE TEST LEAF  - 3	43

12.4 ATP CONFORMANCE REPORT	44

12.5 QTP CONFORMANCE REPORT	45

Annexure – 1	46

ANNEXURE - 2	50

EMI/EMC PROCEDURES	50

EMI/EMC TEST PROCEDURES	51

a.	CE102 Test:-	52

b.	CS101 Test:-	53

c.	CS114 Test:-	54

d.	CS115 Test:-	56

e.	CS116 Test:-	57

f.	RE102 Test:-	58

g.	RS103 Test:-	59

h.	Human Electrostatic Discharge Limit (HESD):-	60

# LIST OF ABBREVIATIONS

| Sl. No | Acronym | Definition |
| --- | --- | --- |
| 1 | ADC | Analog to Digital Converter |
| 2 | DIP | Digital Input |
| 3 | DAC | Digital To Analog Converter |
| 4 | DOP | Digital Output |
| 5 | EMA | Electro Mechanical Actuator |
| 6 | GPS | Global Positioning System |
| 7 | GLONASS | Global Navigation Satellite System |
| 8 | INS | Inertial Navigation System |
| 9 | NGSD | Non ‘G’ Sensitive Drift |
| 10 | IRNSS | Indian Regional Navigational Satellite System |
| 11 | LC | Launch Computer |
| 12 | MMW | Milli-Meter Wave |
| 13 | MAM | Misalignment |
| 14 | MIU | Missile Interface Unit |
| 15 | NVRAM | Non Volatile Random Access Memory |
| 16 | OBC | On Board Computer |
| 17 | RINS | Ring Laser based INS-GPS+GLONASS |
| 18 | RLG | Ring Laser Gyro |
| 19 | SDRAM | Synchronous Random Access Memory |
| 20 | SATNAV | Satellite Navigation |
| 21 | SSPORT | Synchronous Serial PORT |
| 22 | SMPS | Switched Mode Power Supply |
| 23 | SOC | System On Chip |
| 24 | AT | Acceptance Test |
| 25 | QT | Qualification Test |
| 26 | EMI/EMC | Electro-Magnetic Interference/Electro-Magnetic Compatibility |
| 27 | PREET | Pre Environmental Testing |
| 28 | INSET | In Situ Environmental Testing |
| 29 | POET | Post Environmental Testing |
| 30 | UUT | Unit Under Test |
| 31 | EUT | Equipment Under Test |
| 32 | BC | Bus Controller |
| 33 | RT | Remote Terminal |
| 34 | MIL-STD | Military Standard |

# 1.0     SCOPE OF THE DOCUMENT

This Document gives brief  Description ,Specifications, Qualification and Acceptance Test Procedures with test result recording formats and Acceptance Criteria for Integrated Avionics Unit (IAU) for PRALAY.

# 2.0     LIST OF DRAWINGS

| SL.NO | DRAWING NAME | DRAWING NUMBER |
| --- | --- | --- |
| 1 | INS-MK-1 TOP COVER | RCI/DNEC/PED/PMK/020 |
| 2 | PRALAY INS MK-1 CHASSIS SUB ASSY | RCI/DNEC/PED/PLY/INS/MK1/00 |
| 3 | INS MK-1 CHASSIS | RCI/DNEC/PED/PMK/010 |
| 4 | JACK PLATE-2 | RCI/DNEC/PED/PLY/INS/MK1/06 |
| 5 | 6 PIN JACK PLATE | RCI/DNEC/PED/PLY/INS/MK1/10 |
| 6 | ALIGNMENT BUSH | RCI/DNEC/PED/PLY/INS/MK1/08 |
| 7 | ALIGNMENT PIN | RCI/DNEC/PED/PMK/05 |
| 8 | 66 PIN JACK PLATE | RCI/DNEC/PED/PLY/INS/MK1/09 |
| 9 | ALIGNMENT BUSH | RCI/DNEC/PED/PLY/INS/MK1/05 |
| 10 | MOUNTING BUSH | RCI/DNEC/PED/PLY/INS/MK1/06 |
| 11 | 79 PIN JACK PLATE | RCI/DNEC/PED/PLY/INS/MK1/08 |
| 12 | 22 PIN JACK PLATE | RCI/DNEC/PED/PLY/INS/MK1/06 |

# 3.0 LIST OF STANDARDS

| Sl. No. | Description | Standard/Directive No. |
| --- | --- | --- |
|  | Resistors | MIL-PRF-55342 |
|  | Capacitors | MIL-PRF-55365, MIL COTS,MIL PRF,55681 |
|  | Connectors | MIL – DTL – 38999(SERIES III) |
|  | PCBs | IPC 610 F , Class 3 |
|  | EMI/EMC | MIL-STD-461E |

# 4.0 APPLICABLE DOCUMENTS

| Sl. No. | Document Description | Document No. |
| --- | --- | --- |
|  | PRALAY IAU DESIGN DOCUMENT | Doc. No.: RCI/DNEC/PRALAY IAU/01/2016 Date of Issue: 27 /01 / 2016. Rev. No. : 00 |
|  | QT/AT Document | Doc. No.: RCI/PRALAY IAU/QT/AT/2015/04 |
|  | Environmental Test Specification | Doc. No.: RCI/PRALAY IAU/QT/AT Doc/001 Date:28/1/2016 |
|  | INS-GNSS-OBC Interface Control Document | Doc. No.: RCI/DNEC/PRALAY/ICD/01 |

# 5.0 INTRODUCTION

Integrated Avionics Unit (IAU) is used in PRALAY missile system to enhance size, weight and the computational capability. It is an integrated system consisting of OBC, MIU, GPS Receiver & R167 IMU based INS system in a single package. Integrated INS, OBC and MIU system enhances the reliability of the system.

# 6.0 DESCRIPTION

The INS system will be configured with built-in GPS, GLONASS and IRNSS receivers for aiding. It provides Pure, Hybrid and SATNAV navigation solutions simultaneously. The OBC is designed to carry out various On-board Functions i.e., Control, Guidance and In-Flight Mission Sequencing as well as Telemetry Data Posting. It also carries out Health Checks of On-Board Systems during Missile integration as well as Prelaunch Checks. IAU is one of the critical subsystems that is required to function with high reliability in safety critical environment. It communicates with other subsystems on MIL-STD-1553B Bus. It is configured as Bus Controller and issues commands to various sub-systems over MIL-STD-1553B bus as per the mission requirement. IAU also integrates Missile Interface Unit (MIU) that acts as an interface between the On-board computer and the various missile sub-systems. It is used to read various vehicle parameters including the battery voltages, pressure sensor valves, etc and switch On/Off  various subsystems as well as to fire Pyros.

The design is based on two SOC modules with PowerPC- 440 Processor embedded in it and other interfacing electronics catering to the functionality of OBC, INS and MIU. The design consists of four daughter boards to cater for the requirements of OBC, INS, MIU and 2-RF GPS-IRNSS card. The daughter board-1 caters for OBC functionality and consists of SOC, Memories, FPGA and Discrete Outputs. It also consists of communication resources such as MIL-STD-1553B and RS-422. The daughter board-2 caters for MIU functionality and consists of Analog I/O resources (ADC and DAC) and Discrete Inputs. The daughter board-3 caters for INS functionality and consists of SOC, Memories, FPGA and I/Os. All four daughter boards are interfaced to the Mother Board which is a Rigid-Flex-Rigid PCB on which all internal and external connectors are mounted.  The Input and Output Signals are brought to the connectors through flex PCB. All four daughter boards are rigid PCBs. These are mounted on Mother Board through Hypertac Connectors.

The required supply voltages are generated by a DC-to-DC converter mounted on chassis with input of 28V supply. It converts the given Input Voltage into 6 different DC Output Voltages.  The Output Voltages of the converter are given to the unit through Airborne Connector. The DC-DC converter has built-in EMI/EMC filters. 28V supply from external power connector is directly routed to IMU as there is a DC-DC convertor inside IMU. The following figure shows the block diagram and inter-connectivity details of the unit.

# 7.0 SPECIFICATIONS

## 7.1 Specifications of OBC Card:

| Parameter | Description |
| --- | --- |
| Processor | PPC440 Embedded In SOC Compute |
| Memory | FLASH 16MB [4M X 32bit] SRAM 4MB [2M X 32bit] NVRAM 512KBPROM EEPROM 32M Bit |
| FPGA | Xilinx SPARTAN-6 |
| 1553 Channels | 3MIL-STD-1553B Channels From SOC |
| Serial Channel | 3 Asynchronous RS422 Serial Channels From SOC |
| Ethernet | 1 Ethernet Channel Of 10/100 Mbps |
| DIPS | 24 Nos Input Voltage : 28 V or 5 V |
| No. Of. Layers | 18 |
| PCB Dimensions | 140 x 123 x 2.4 mm |

## 7.2 Specifications of MIU Card:

| Parameter | Description |
| --- | --- |
| FPGA | SPARTAN-6 |
| Analog Outputs | 8 Nos Single ended Dynamic Range : ±10V |
| Analog Inputs | 8 Nos – Differential Dynamic range: ±10V |
| Single Ended Inputs | 16 Nos Dynamic range: ±10V |
| DOPS | 20 |
| No. Of. Layers | 12 |
| PCB Dimensions | 140 x 123 x 1.6 mm |

## 7.3 Specifications of INS Card:

| Parameter | Description |
| --- | --- |
| Processor | SOC Compute With PowerPC 440 Processor Core Operating Up 400mhz With 32KB I- And D-Caches |
| Memory | FLASH 16MB [4M X 32bit] SRAM 4MB [2M X 32bit] NVRAM 512KB EEPROM 32M Bit |
| FPGA | SPARTAN_6 |
| 1553 Channels | 1 MIL-STD-1553B Channel Implemented In FPGA As IP Core |
| Serial Channel | 2 Asynchronous RS422 Serial Channels From SOC |
| Ethernet | 1 Ethernet Channel Of 10/100 Mbps |
| No. Of. Layers | 16 |
| PCB Dimensions | 140 x 123 x 2.0 mm |

## 7.4 Specifications of GPS+GLONASS/IRNSS RECEIVER

- Number of Channels			             :	24-(12 GPS and 12 GLONASS/IRNSS)

- Number of  RF Channels			:	2

- Time to first fix				             :	48 Sec without Almanac & Ephemeris

  - Accuracy : ( 1 sigma, without

Ionosphere & Clock errors

and DOP < 6.0)

Position					:	30 m (RMS)

Velocity					:	0.3 m/sec (RMS)

  - Dynamics:

Velocity					:	6000 m/sec

Acceleration				:	25 g

Jerk				             :	10 g/s

- Reacquisition 				:	5 secs

- Navigation solution

Mode					:	3D Position, velocity & Time

Update rate				             :	1 Hz

- Host communication

Interface				:	RS-232 compatible, MIL -1553B

Baud Rate				:	57600 Bps

- Environmental characteristics

Operational Temperature Range		:	-30°C to +65°C

Storage Temperature Range		             :	-50°C to +85°C

- Electrical Characteristics

Power supply voltage			:	5.0 V (±250 mV)

Power supply voltage			:	5 W (typical)

- Connectors

RF						:	SMA Connector

Interface	  :	65 pin Connectors

(KA65_127 BPM)

- Provision for serial Upgrade		              :	Should be Provided

## 7.5  Qualification Tests Specifications

| Sl. No | TEST | Description of Test |  | Electrical tests to be conducted | Conforming / Non-conforming | OBSERVED RESULT |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Burn In Test | + 65° C for 168 hours |  | The PCBs/modules/units will be soaked at + 65° C for 168 hours in power ON condition |  |  |
| 2 | Initial performance checks | Functional checks, isolation & continuity checks |  |  |  |  |
|  | QUALIFICATION TESTS |  |  |  |  |  |
| 1 | ESS Test Specifications | Vibration (QT Level) | 20Hz-100Hz:+6dB/Oct, 100Hz-1000Hz:0.045g2/Hz, 1000Hz-2000Hz:-6dB/Oct, Overall Grams :8.03, Duration:270 sec No.Of.Axis:3 |  |  |  |
| 2 |  | Thermal Cycling | +70°C(10 min. soaking),dwell time:1Hr -30°C(10 min. soaking),dwell time:1Hr Rate of Change Of Temp:10°C/min No. of. Cycles: 8 |  |  |  |
| 3 |  | Vibration (QT Level) | 20Hz-100Hz:+6dB/Oct, 100Hz-1000Hz:0.045g2/Hz, 1000Hz-2000Hz:-6dB/Oct, Overall Grams :8.03, Duration:270 sec No.Of.Axis:3 |  |  |  |
| 4 | Tropical Exposure |  | 45°C+2°C,95% RH for 12Hrs No. of. Cycles : 14 Total Cycle Time : 24Hrs |  |  |  |
| 5 | High Altitude |  | Altitude : 50Km Temperature :+40°C Duration:15minutes (<1 millibar) |  |  |  |
| 6 | Acceleration |  | Level:18g Duration:60sec, Direction: All 3 Axis and 6 Directions |  |  |  |
| 7 | Shock |  | Half Sine Pulse 40g For 11 Milliseconds 3 Shocks Along Each Axis & Direction |  |  |  |
| 8 | Bump |  | 4000 Bumps,(40g) 2-3 Bumps/Sec Pulse Width :6 Milli Seconds |  |  |  |
| 9 | Mould Growth |  | On test coupons – conformal coated PCB's, Insulations, EMI Gaskets, Rubber |  |  |  |
| 10 | Corrosion (Saline Atmosphere) |  | 10 minutes salt spray 24Hrs,Soaking at 35°C, 90%RH, 10Cycles On test coupons –chassis of equipment, passivated fasteners, painted ports |  |  |  |

## 7.6   Acceptance Tests Specifications

| ESS Vibration(AT Level) Thermal Cycling Vibration(AT Level) Shock Damp Heat Sl. No | TEST | Description of Test |  | Electrical tests to be conducted | Conforming / Non-conforming | OBSERVED RESULT |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Burn In Test | + 65° C for 168 hours |  | The PCBs/modules/units will be soaked at + 65° C for 168 hours in power ON condition |  |  |
| 2 | Initial performance checks | Functional checks, isolation & continuity checks |  |  |  |  |
|  | ACCEPTANCE TESTS |  |  |  |  |  |
| 1 | ESS Test Specifications | Vibration Test (AT Level) | 20Hz-100Hz:+6dB/Oct, 100Hz-1000Hz:0.03g2/Hz, 1000Hz-2000Hz:-6dB/Oct, Overall Grams :6.56 Duration:180sec No.Of.Axis:3 |  |  |  |
| 2 |  | Thermal Cycling | +70°C(10 min. soaking),dwell time:1Hr -30°C(10 min. soaking),dwell time:1Hr Rate of Change Of Temp:10°C/min No. of. Cycles: 8 |  |  |  |
| 3 |  | Vibration Test (AT Level) | 20Hz-100Hz:+6dB/Oct,100Hz-1000Hz:0.03g2/Hz, 1000Hz-2000Hz:-6dB/Oct, Overall Grams :6.56 Duration:180sec No.Of.Axis:3 |  |  |  |
| 4 | Damp Heat |  | 45ºC+2°C for 8 Hrs . Relative Humidity :95% |  |  |  |
| 5 | Shock |  | 20g for 11 milliseconds, Half Sine Pulse 3 shocks along each axis and direction |  |  |  |
| 6 | EMI/EMC: (MIL-STD-461-C) |  | CS 115: Conducted Susceptibility, EUT is subjected to pre-calibrated signal having rise and fall time <= 2nSec, pulse width 30nSec, amplitude 5Amps at 30Hz rate for 60 seconds on i) Bus cable ,DIP-DOP cable ii) 28V DC High, Low Line iii) Power Cable bunch RS 103: 20V/m, Radiated Susceptibility, with pulse modularization of 1KHz in the following frequency bands on both vertical polarization & horizontal polarization. - 2 – 100MHz - 30 – 100MHz - 100 – 1000MHz - 1GHz - 2.5GHz - 2.5GHz – 8GHz - 8GHz – 18GHz |  |  |  |
| 7 | Final performance checks |  | Functional checks, isolation & continuity checks |  |  |  |

## 7.7  EMI/EMC as per MIL – STD  - 461 - E Specifications

| SL NO | TEST | TEST DESCRIPTION | APPLICABILITY/LIMITLINE/ TEST PROCEDURE | TEST DURATION | TEST METHOD |
| --- | --- | --- | --- | --- | --- |
| 1 | HESD | Human Electrostatic Discharge Test. | Applicability: Applicable to all interfacing connector is mounted on the equipment and chassis. Test setup and test procedure, As per mil-std-461E | Two pulses shall bed is charged on each of connector and EUT chassis with a time interval of 5 seconds between the pulse. | PREET, INSET, POET |
| 2 | CE102 | Conducted Emission Power leads 10KHz to 10MHz | Applicability : This requirement is applicable from 10KHz to 10MHz for all power leads . | Two pulses shall bed charged on each connector with a time interval of 10 seconds between the pulse. | PREET, INSET, POET |
| 3 | CS101 | Conducted Susceptibility Power range extending 150KHz | Applicability :This is applicable to equipment and subsystem AC & DC input power .This is applicable over the frequency range of 30Hz to 150KHz. | Two pulses shall bed charged on each connector with a time interval of 10 seconds between the pulse. | PREET, INSET, POET |
| 4 | CS114 | Conducted Susceptibility | This requirement is applicable to all interconnecting cables and power cables . | Two pulses shall bed charged on each connector with a time interval of 10 seconds between the pulse. | PREET, INSET, POET |
| 5 | CS115 | Conducted Susceptibility | This requirement is applicable to all aircraft, space and ground system interconnecting cable including power cables. It is also applicable for surface ship and submarine subsystems and equipment when specified by the procuring activity. | Two pulses shall bed charged on each connector with a time interval of 10 seconds between the pulse. | PREET, INSET, POET |
| 6 | CS116 | Conducted Susceptibility | This is applicable to all interconnecting cables including power cables and individual high side power leads . Power returns and neutrals need not be tested individually. | Two pulses shall bed charged on each connector with a time interval of 10 seconds between the pulse. | PREET, INSET, POET |
| 7 | RE102 | Radiated Emission | This test is used to verify that electric field emissions from the EUT and its associated cabling do not exceed specified requirements. The basic intent of the requirement is to protect sensitive receivers from interference coupled through the antennas associated with the receiver. | Two pulses shall bed charged on each connector with a time interval of 10 seconds between the pulse. | PREET, INSET, POET |
| 8 | RS103 | Radiated Susceptibility | This requirement is applicable to equipment and subsystem enclosures and all interconnecting cables. The requirement at the tuned frequency of an antenna – connected receiver is 20Db above the RE102 limit associated with the particular platform application. | Two pulses shall bed charged on each connector with a time interval of 10 seconds between the pulse. | PREET, INSET, POET |

## 8.0 LIST OF DELIVERABLES

| Sl .No | Description | Qty. | Acceptance Method |
| --- | --- | --- | --- |
| 1 | IAU |  | QT |
| 2 | IAU |  | AT |
| 3 | IAU QC reports as per Process Document |  | QC/Process Document |
| 4 | IAU QT/AT reports as per QT/AT Document |  | QT/AT Document |

- PCB group-B Certificates

- Component conformance and Screening Certificates

- Mechanical Housing ,raw material and dimensional inspection reports

## 9.0 READINESS OF QT/AT

Before commencement of testing the following aspects shall be reviewed:

- Readiness of the Test Equipment /facilities.

- Readiness of approved  AT document

- Readiness of Material, Assemblies, QC reports & Test reports as per Approved Process Document.

- Availability of Log Book.

## 9.1 READINESS OF TEST EQUIPMENT/FACILITIES

The Test Setup used for checking the functionality of the unit is referred as “PRALAY IAU  TEST SETUP”. This is used for clearing the unit during environmental tests and functional tests at lab. All the cables are tested by checking the continuity.

    - READINESS OF MATERIAL,ASSEMBLIES,QC & TEST REPORTS AS PER PROCESS DOCUMENT

The following points are to be followed for proper packing and handling:

- Marking/Identification:

- Each PRALAY IAU  unit is marked with the designation no.

- Marking of the unit is made with an engraving.

- The engraving is on to the device.

PACKAGE NAME                       :

PACKAGE SL NO	                   :

MFG	BY    		                   :

PACKAGE MODEL NO              :

The following information is engraved on to the plate.

The connectors are also marked with the help of engraving. Connectors are marked. PRALAY IAU connectors are closed with plastic caps. All the connectors of the Test Jig cables are covered with plastic cover.  The material, assembly, QC report and PWB & test reports as per process Document have to be verified.

## 9.2 Safety Measures

Dangerous voltages for human life are used during operation with OBC. To assure safety of the personnel working with the OBC during their operation and repair the following requirements should be observed.

- The working place should be grounded

- It is prohibited to replace removable components and solder the connectors under voltage.

- It is necessary to De – Energize the electrical circuits and check absence of voltage by voltmeter before the checking the electrical circuits.

- The personnel servicing power supply system should be trained in procedures to help a person suffering from electrical shock, also first medical aid regulations and methods to put out a fire.

- The switches commutating supply voltages ,test jacks +28v and -28v,connector d1are the dangerous parts on the IAU. Disconnect the power cable before removal of the IAU covers.

- If the service personnel strictly observe the above mentioned precautions , the operation with the IAU will be safe  for the personnel.

- The working place for checking the IAU  should be fitted with voltage source(28 ± 4)v, and power of not less than 150W.

## 9.3 Availability of logbook

A logbook is prepared for each and every IAU unit which contains Process Sheets of CPU module ,BUS module, Mother Board and connector Block. Bill of Material of all the modules is also included for module level inspection. All the Process Sheets starting from PCB inspection till final assembly are to be filled up and approved by R&QA.

# 10 SEQUENCE OF TESTS

## 10.1 Qualification Tests

| S. No. | Test | PREET | INSET | POET | PARAMETERS MONITORED |
| --- | --- | --- | --- | --- | --- |
| 1 | Initial Performance Checks | NA | NA | NA | Cold Checks, Continuity & IR Checks, I/Os and resource checks at Voltages 24 V, 28 V, 32 V DC, Measurement of Telemetry monitoring signals |
| 2 | Vibration (QT Level) | A | A | A | IO & Resources check at 28 V |
| 3 | Thermal Cycling | A | A | A | IO & Resources check at 28 V |
| 4 | Vibration (QT Level) | A | A | A | IO & Resources check at 28 V |
| 5 | Tropical Exposure | A | A | A | As Above |
| 6 | High Altitude | A | A | A | As Above |
| 7 | Acceleration | A | A | A | Same as initial performance check |
| 8 | Shock | A | A | A | As Above |
| 9 | Bump | A | A | A | As Above |
| 10 | Mould Growth | A | A | A | As Above |
| 11 | Corrosion | A | A | A | As Above |

## 10.2 Acceptance Tests

| S. No. | Test | PREET | INSET | POET | PARAMETERS MONITORED |
| --- | --- | --- | --- | --- | --- |
|  | Initial Performance Checks | NA | NA | NA | Cold Checks, Continuity & IR Checks, I/Os and resource checks at Voltages 24 V, 28 V, 32 V DC, Measurement of Telemetry monitoring signals |
|  | Vibration(AT Level) | A | A | A | IO & Resources check at 28 V |
|  | Thermal Cycling | A | A | A | As Above |
|  | Vibration(AT Level) | A | A | A | As Above |
|  | Damp Heat | A | A | A | As Above |
|  | Shock | A | A | A | As Above |
|  | EMI/EMC (CS115,RS103) | NA | A | NA | As Above |

# 11. TEST PROCEDURES

## 11.1 Functional Test Procedure

To carry out the all functional checks of the UUT, various test cases are generated. All functional checks are carried out in the test condition with the support of various measuring source equipment and test jig. During the complete AT test cycle, product needs be subjected to various environmental tests. Full functional checks needs to carry out on UUT before and after subjecting to environmental test; for certain environmental tests UUT need to be tested for performance during test condition. After completion of the entire environmental test, functional checks need to be carried out on the system.  INS module of PRALAY IAU is configured as Remote Terminal and OBC module is configured as Bus Controller on Power-on.  All the resources in the unit are tested periodically and the results are sent through 1553B and RS422communication links.  This is a cyclic test which continues until the test is stopped. The results are logged in the Host PC for verification. The INS data is transmitted on 1553 bus and logged in host PC for verification. The Test Setup used for checking the functionality of the unit and is referred as “IAU TEST SETUP”. This is used for clearing the unit during environmental tests and functional tests at lab. All the cables are tested by checking the continuity.

The objective is to test all the resources of IAU. The main resources of  IAU to be checked are SRAM, Non-Volatile RAM (NVRAM), 1553 BUS0 RAM, 1553 BUS1 RAM, 1553 BUS communication, FLASH memory, ADC & DAC channels, DIP & DOP channels,  RS422, GPS test, Navigation test and IMU data test.

IAU has three modes of operation-

Flash Loader Mode : For entering into Flash Loader Mode, 1553 message should be sent from HOST PC before Unit Power-On.

ENTEST Mode : For entering into ENTEST Mode, character ‘e’ should be given from keyboard of HOST PC.

Mission Mode : This is default mode.

After Power-On, on entering into ENTEST mode, IAU performs following activities:

- Initializes all its resources

- OBC Configures M1 bus as Bus Controller and M2 as Remote terminal and INS configures M3 bus as RT

- Performs self tests of all its resources  and displays POST result on Hyper terminal.

- INS goes into navigation mode and waits for 1553 messages from Host PC.

- OBC starts cyclic test of all resources.

- OBC posts test results and INS data on M1 bus to Host PC for  verification. The Cyclic test results in the form of alphabets are also send on RS422 channel 1 to Host PC and are displayed on Hyper terminal. Upper Case Alphabet indicates test pass and Lower Case  Alphabet indicates test fail.

- During Environmental tests, IAU should switch over to ENTEST mode. In ENTEST mode, the results of resource checks are sent through serial port and MIL-STD-1553B.

The resource of IAM checked and displayed are given below

| Sl. No. | Name of Parameter Under Check | Procedure | Expected Data | Expected Result |
| --- | --- | --- | --- | --- |
|  | SIO port loop back test | OBC SIO-2 and OBC SIO-3 are Looped back.INS SIO-1 Tx & Rx are Looped back | T/t | PASS/ FAIL |
|  | SRAM Test | Walking ones, Walking zeros and Pattern tests | S/s | PASS/ FAIL |
|  | NVRAM Test | Walking ones, Walking zeros and Pattern tests, Retention test | N/n | PASS/ FAIL |
|  | B0 RAM test | Walking ones, Walking zeros and Pattern tests | W/w | PASS/ FAIL |
|  | B1 RAM test | Walking ones, Walking zeros and Pattern tests | X/x | PASS/ FAIL |
|  | ADC-DAC loop back test | The DAC and ADC channels are externally looped back and voltages varying from -10V to 10V are applied to ADC channels by setting DAC channels | P/p | PASS/ FAIL |
|  | DOP-DIP loop back test | Each DIP channel and DOP channels of IAU are checked with the help of Relays in relay Box. Upon receiving a DOP command from IAU, relay is operated and status is read using DIPs. | E/e | PASS/ FAIL |
|  | B0 & B1 BUS BC RT Test (CH-A) | B0 is initialized as BC and B1 as RT Messages are sent and then read back and verified. | A/a | PASS/ FAIL |
|  | B0 & B1 BUS BC RT Test (CH-B) | B0 is initialized as BC and B1 as RT Messages are sent and then read back and verified. | B/b | PASS/ FAIL |
|  | FLASH Checksum | Carried out once before start of Cyclic test. Calculated Checksum is compared with Stored checksum and result is sent. | M/m | PASS/ FAIL |

If all these tests are PASSED  and  INS  data is o.k,  then the IAU performance is satisfactory.

### Power On Self Test for OBC Resources:-

| S.NO | TEST NAME | EXPECTED RESULT | REMARKS |
| --- | --- | --- | --- |
| 1 | SRAM Test | Result Is Displayed In Hyper Terminal |  |
| 2 | BUS0 RAM Test |  |  |
| 3 | BUS1 RAM Test |  |  |
| 4 | Flash Checksum Test |  |  |
| 5 | NVRAM Test |  |  |
| 6 | RTC Timer Test |  |  |
| 7 | ADC DAC Interrupt loopback Test |  |  |
| 8 | Interrupt Test |  |  |

## 11.2  Test Set – Up

List Of Test Equipment

The list of test equipment for carrying out functional tests is given below.

- Power Supply:

- Laptop & Accessories:

- Laptop

- Reset Board

- Cables

- RSTB Power (Reset Board Power Cable)

- IAU Power Cable

### 11.2.1  Test  Setup  Procedure

To make the test setup, follow the steps

Check the continuity of each cable as per the cable details and connector details given in Appendix-A. For making the setup follow the steps from step 1 to step 8, referring to the fig: 5

Step 1: Connect the power lines of  IAU from 6 pin connector to power supply. Connect power lines of reset board to power supply.

Step 2: Make the loopback connections of ADCs and DACs by mating the 37 pin D-sub connectors as shown in fig: 5.

Step 3:	 Make the loopback connections of SIO Tx and Rx.

Step 4: Connect DIP connector from 79 I/O pin connector of IAU to the DIP connector of the Relay Box as shown in figure 4.

Step 5:	Connect DOPs  of  IAU  from 79  I/O pin connector to the DOP connector of the Relay Box

Step 6: Connect SIO and Reset lines 79 pin connector of OBC from 66 I/O pin connector to the 25 pin connector of Reset board.

Step 7:	 Connect cable from reset board to host PC.

Step 8:  Connect the Bus cables as shown in figure

11.3 Initial And Final Performance Checks

The following resources have to be checked initially and finally at 24V, 28V, 32V

- Current Measurement

- Cold Checks

| Sl. No. | Name of Parameter Under Check | Expected Result PASS/FAIL | Observations |  |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 24V | 28V | 32V |  |
| 1 | SRAM Test | S/s |  |  |  |  |
| 2 | ADC-DAC loopback Test | P/p |  |  |  |  |
| 3 | NVRAM check | N/n |  |  |  |  |
| 4 | B0 RAM check | W/w |  |  |  |  |
| 5 | B1 RAM check | X/x |  |  |  |  |
| 6 | BC RT checks(ch-A) | AB/ab |  |  |  |  |
| 7 | BC RT checks(ch-B) | AB/ab |  |  |  |  |
| 8 | SIO loopback Test | T/t |  |  |  |  |
| 9 | DIP-DOP test | E/e |  |  |  |  |

The following results to be verified for validating IAU’s performance

## 11.4 Qualification Tests

Qualification tests shall be carried out for design verification to demonstrate the capability of subsystem to with stand the severe conditions with specified margins of safety.One package shall undergo qualification tests.

The Qualification test specifications are given in section.7.2. The Qualification test sequence is given in Fig.4.0. The Qualification test procedures are given in this section below.

## 11.5 Acceptance Tests

The acceptance tests are performance verification tests on packages, Acceptance Testing demonstrates the performance of PRALAY IAU under environmental conditions. This testing locates material, workmanship, deficiencies in a proven design. This test is conducted on all units.

The Acceptance test specifications are given in Section.7.3. The Acceptance test sequence is given in Fig.5.0. The Acceptance test procedures are given in this section below.

## 11.6 Burn-In Test

Purpose

PCBs shall be subjected to 168  hours of Burn-in test at 65ºC.

Test Objectives

The primary objective of the test is to ensure that components and PCB can with stand high temperature test.

Test Facility

The required test facility consists of a chamber which controls temperature, chamber should support maximum required temperature of 65ºC

Test specifications

PCBs are kept in a chamber with ambient temperature set to 65ºC for duration of 168 hours; performance check is carried at every five hours. Last three hours shall be defect free.

Procedure

Step1:	     This test is conducted with temporary integration/PCB level in power-on condition

Step2:     Tested PCBs are kept in the thermal chamber. All arrangements shall be made externally to carry out PCB screening performance checks at every five hours period.

Step3:      Power on the system and check the performance of the PCB at every 5 hours period.

Step4:	     Temperature of the chamber is raised to 65ºC, with tolerance of +5ºC/ 0ºC.

Step5:	     PCBs shall be maintained at this temperature continuously for 168 hours in power ON condition.

Step6:	      In case of failure, the components shall be replaced by components which have gone and passed High Temperature storage and thermal shock before assembly and continue the test for remaining hours. If the failure occurs after the 166th /167th /168th hour, expose the PCB for further 3 hours to confirm the adequacy of the repair work.

Step7:       On completion of Burn-in test, switch OFF the chamber.

Test analysis

Carry out visual inspection using 30X magnifier, followed by board level performance check. In case of failure, analyze the defect in consultation with Certification/Quality Assurance Agency.

## 11.7 Vibration Test

Objective:

To bring out the workmanship and latent defects.

Test Severities:

Acceptance Test:-

20Hz – 100Hz:+6dB/Oct, 100Hz – 1000Hz:0.03g2/Hz, 1000Hz – 2000Hz:-6dB/Oct,

Overall Grams:6.56,Duration :180sec ,No.Of.Axis :3

Qualification Test:-

20Hz – 100Hz:+6dB/Oct, 100Hz – 1000Hz:0.045g2/Hz, 1000Hz – 2000Hz:-6dB/Oct,

Overall Grams:8.03,Duration :270sec ,No.Of.Axis :3

Test Procedure

- Fix the unit on the fixture properly as per the values specified, perform a visual inspection of the unit for any physical defects and check the connectors are mated properly or not then take the PREET readings as per the test procedure

- Apply specified vibration level to the test item/fixture interface.

- Monitor vibration levels and unit performance as per the functional test procedure given continuously throughout the test and take the INSET readings. If levels shift or a failure occurs, stop the test. Analyze the reason for the shift and take the necessary action.

- If the required duration has been achieved, stop the vibration.

- Visually examine the equipment and take the POET readings as per the functional test procedure

- Repeat steps 1 to 5 along other axes.

- NOTE: Note down the PREET INSET and POET test reading it the functional test leaf for ENTEST given in Appendix B

## 11.8  Thermal Cycling Test

Objective:

To bring out the workmanship and latent defects in the test item by subjecting it to rapid temperature cycling stress.

Test Severities:

+70°C(10 min. soaking),Dwell Time:1Hr,-30°C(10 min. soaking),Dwell Time:1Hr,

Rate of Change Of Temp:10°C/Min, No. of. Cycles: 8

Test Procedure:

- Keep the unit inside the thermal chamber and mate the connectors as per the drawing,

- Take the PREET readings as per the functional test procedure

- Switch “ON” the chamber and unit and then increase the temp to 70°C. The rate of change of temperature shall be 10°C/minute.

- Maintain this specified high temperature for 1 hour, carry out the functional test as per the procedure and take the INSET readings in last 15 minutes duration.

- After 1 hour switch OFF the Equipment.

- Allow the temperature of the chamber to fall to -30°C. The rate of change of temperature shall be 10°C/minute.

- Maintain this temperature for 1 hour.

- During end of this period, switch ON the Equipment; carry out the functional test as per test procedure, and take the INSET readings in last 15 minutes duration.

- After 1 hour the temperature of the chamber shall be increased to 70°C while keeping the Equipment in switched ON.

- Up to this constitutes one Cycle. Similarly, follow for remaining cycles.

- Then Switch “OFF” the chamber and bring it to the room temperature.

- After attaining the Room temperature, visually examine the Equipment and Switch “ON” the Equipment and take the POET readings as per the functional test procedure.

## 11.9  Damp Heat

Objective:    To determine the suitability of equipment for use and storage Conditions of                                                                            high humidity when combined with cycle temperature changes.

Test Severities:

Acceptance Test : 45ºC+2°C for 8 Hrs. Relative Humidity :95%

Test Procedure

- Keep the unit in the humidity chamber and mate the connectors as per the drawing.

- Take the PREET readings as per the functional test procedure given in chapter No.: 9.5.3

- The temperature and RH of the chamber shall be raised to 450C and 95% respectively.

- Maintain this condition as per the specifications.

- Switch ON the equipment and operate it when the chamber condition is 450C + 20C

(With RH 95%) and take the INSET readings as per the functional test procedure

- After completion of the period bring the chamber conditions to room condition & after reaching the room conditions Switch off the chamber.

- Take the POET readings as per the functional test procedure

## 11.10  Shock Test

Objective:

To determine the ability of the equipment to survive the effects of non-repetitive mechanical shocks.

Test Severities:

Acceptance Tests: 20g for 11m sec half sine wave 3 shocks in each of 6 directions.

Qualification Tests: 40g for 11m sec half sine wave 3 shocks in each of 6 directions

Test procedure:

- Fix the unit on the fixture properly as per the torque values specified, perform a visual inspection of the unit for any physical defects and check the connectors are mated properly or not then take the PREET readings as per the test procedure

- Switch ON the unit apply 20g shock, half sine wave, three shocks, 11 milliseconds along X+ direction. Take INSET readings as per the functional test procedure

- The unit shall be visually examined and take the POET readings as per the test procedure

- Repeat steps from 1 to 3 for other directions  (X+,X-,Y+,Y-,Z+,Z-)

- NOTE: Note down the PREET, INSET and POET test readings in the functional test leaf for ENTEST given in appendix B

Half Sine:-

## 11.11 Bump Test

Objective:

To determine the ability of electronic and electrical equipment to withstand repeated bumps without malfunctioning and mechanical damage.

Test Severities:

Acceptance Test: 400 bumps(40g), Pulse duration 6 ± 1msec, Peak acceleration 250 m/s2 Rate of bumps 2-3 bumps/sec.

Qualification Test: 4000 bumps(40g), Pulse duration 6 ± 1msec, Peak acceleration 250 m/s2 Rate of bumps 2-3 bumps/sec.

Test Procedure:

- Initially the equipment shall be visually examined and shall be electrically and                  mechanically checked as specified.

- The unit under test is switched off & packed condition.

- The unit is fixed on the setup.

- Specified condition is applied on the packed unit.

- Unpack the unit after test.

- Examine the unit visually and then test the unit electrically for functional performance.

## 11.12 Tropical Exposure

Objective:

To determine the suitability of equipment for use and storage Conditions of                                                                    high humidity when combined with cycle temperature changes.

Test Severities:

Qualification Test: 45°C + 2°C, 95% RH for 12Hrs, No. of. Cycles: 14,total cycle time :24Hrs.

Test Procedure:

1. Keep the unit in the humidity chamber and mate the connectors as per the drawing.

2. Take the PREET readings as per the functional test procedure given in chapter No.: 9.5.3

3. The temperature and RH of the chamber shall be raised to 450C and 95% respectively.

4. Maintain this condition as per the specifications.

5. Switch ON the equipment and operate it when the chamber condition is 450C + 20C

(With RH 95%) and take the INSET readings as per the functional test procedure

6. After completion of the period bring the chamber conditions to room condition & after reaching the room conditions Switch off the chamber.

7. Take the POET readings as per the functional test procedure

Temp.

## 11.13 Acceleration

Objective:

To determine the structural integrity and satisfactory performance of equipment during and after subjection to a steady acceleration.

Test Severities:

Qualification Test : Level:18g , Duration:60sec, Direction: All 3 Axis and 6 Directions

Test Procedure:

- Fix the unit on the fixture of the centrifuge table.

- The equipment shall be visually examined and take the PREET readings as per the test procedure .The unit shall be subjected to the acceleration as per the specifications.

- Perform the INSET readings as per the functional test procedure given

- After the unit subjected to above tests, finally the unit will be visually examined and take the POET readings as per the test procedure.

- Repeat from 1 to 4 for other directions.

- NOTE: Note down the PREET ,INSET and POET test reading it the functional test leaf for ENTEST given in Appendix B.

# 12  FORMATS

## 12.1 Unit Configuration

Unit Name                                             :

Unit Model No                                      :

Unit Serial No                                       :

Project                                                   :

Manufacturer /Work Center                 :

QT/AT Document Reference               :

Process (QC) Document Reference     :

Date                                                      :

| S.No | Sub Unit | Qty. | Sub Unit Serial No |
| --- | --- | --- | --- |
| 1 |  |  |  |
| 2 |  |  |  |
| 3 |  |  |  |
| 4 |  |  |  |
| 5 |  |  |  |
| 6 |  |  |  |

WORK CENTER                                                                         R&QA

## 12.2 Quality Assessment Report

Unit Name                                             :

Unit Model No                                      :

Unit Serial No                                       :

Project                                                   :

Manufacturer /Work Center                 :

QT/AT Document Reference               :

Date                                                      :

| S. No | Attribute | Relevant Doc/Drawing Nos. With Date | OK/NOT OK/ Remarks |
| --- | --- | --- | --- |
| 1 |  |  |  |
| 2 |  |  |  |
| 3 |  |  |  |
| 4 |  |  |  |
| 5 |  |  |  |
| 6 |  |  |  |

Observations:

Cleared for QT/AT   /   Not Cleared for QT/AT

WORK CENTER                                                                         R&QA

## 12.3 ACCEPTANCE TEST REPORT FORMATS

### ACCEPTANCE TEST LEAF  - 1

Unit Name                                             :

Unit Model No                                      :

Unit Serial No                                       :

Project                                                   :

Class of Test                                         :       Power Checks, Initial/Final Checks

Date                                                      :

| Parameter | Current |  | Measured |  |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
|  | Max | Min | 24V | 28V | 32V |  |
| Current on 28V |  |  |  |  |  |  |

Cold Checks:

| Terminals | Limit | Measured | Remarks |
| --- | --- | --- | --- |
| Resistance Between 28V Low (X1-2,4)Pins |  |  |  |
| Resistance Between 28V High (X1-1,3)Pins |  |  |  |
| Resistance Between 28V High (X1-1,3)Pins And 28V Low (X1-2,4)Pins |  |  |  |
| Resistance Between 28V Low (X1-2,4)Pins And Chassis |  |  |  |
| Resistance Between 28V High (X1-1,3)Pins And Chassis |  |  |  |
| Resistance Between All Connector Pins And Chassis |  |  |  |

Observations:

Power Checks Test Passed/Failed

WORK CENTER                                                                         R&QA

## ACCEPTANCE TEST LEAF  - 2

Unit Name                                             :

Unit Model No                                      :

Unit Serial No                                       :

Project                                                   :

Class of Test                                         :        Initial/ Final Functional  Checks

Date                                                      :

| Sl. No | Description of Tests | Status of test | 24V | 28V | 32V |
| --- | --- | --- | --- | --- | --- |
| 1 | Initial Performance Checks |  |  |  |  |
| 2 | Vibration(AT Level) |  |  |  |  |
| 3 | Thermal Cycling |  |  |  |  |
| 4 | Vibration(AT Level) |  |  |  |  |
| 5 | Damp Heat |  |  |  |  |
| 6 | Shock |  |  |  |  |
| 7 | EMI/EMC (CS115,RS103) |  |  |  |  |
| 8 | Final Performance Checks |  |  |  |  |

Observations:

Initial Functional Test Passed/Failed

WORK CENTER                                                                         R&QA

## ACCEPTANCE TEST LEAF  - 3

Unit Name                                             :

Unit Model No                                      :

Unit Serial No                                       :

Project                                                   :

Class of Test                                         :        Environmental

Date                                                      :

| Sl. No | Description of Tests | Status of test | 24V | 28V | 32V |
| --- | --- | --- | --- | --- | --- |
| 1 | Initial Performance Checks |  |  |  |  |
| 2 | Vibration(AT Level) |  |  |  |  |
| 3 | Thermal Cycling |  |  |  |  |
| 4 | Vibration(AT Level) |  |  |  |  |
| 5 | Damp Heat |  |  |  |  |
| 6 | Shock |  |  |  |  |
| 7 | EMI/EMC (CS115,RS103) |  |  |  |  |
| 8 | Final Performance Checks |  |  |  |  |

Observations:

Unit Under Test Passed/Failed

WORK CENTER                                                                         R&QA

## 12.4 ATP CONFORMANCE REPORT

Unit Name                                              :

Unit Model No                                       :

Unit Serial No                                        :

Project                                                    :

Date                                                        :

QT/AT Document Ref                           :

Amendments Implemented(References) :

| Sl. No | Description of Tests | Conforming/ Non-Conforming | Remarks |
| --- | --- | --- | --- |
| 1 | Initial Performance Checks |  |  |
| 2 | Vibration(AT Level) |  |  |
| 3 | Thermal Cycling |  |  |
| 4 | Vibration(AT Level) |  |  |
| 5 | Damp Heat |  |  |
| 6 | Shock |  |  |
| 7 | EMI/EMC (CS115,RS103) |  |  |
| 8 | Final Performance Checks |  |  |

Observations:

OBC Unit Cleared /Not Cleared

WORK CENTER                                                                         R&QA

## 12.5 QTP CONFORMANCE REPORT

Unit Name                                              :

Unit Model No                                       :

Unit Serial No                                        :

Project                                                    :

Date                                                        :

QT/AT Document Ref                           :

Amendments Implemented(References) :

| Sl. No | Description of Tests | Conforming/ Non-Conforming | Remarks |
| --- | --- | --- | --- |
| 1 | Initial Performance Checks |  |  |
| 2 | Vibration (QT Level) |  |  |
| 3 | Thermal Cycling |  |  |
| 4 | Vibration (QT Level) |  |  |
| 5 | Tropical Exposure |  |  |
| 6 | High Altitude |  |  |
| 7 | Acceleration |  |  |
| 8 | Shock |  |  |
| 9 | Bump |  |  |
| 10 | Mould Growth |  |  |
| 11 | Corrosion |  |  |
| 12 | Final Performance Checks |  |  |

Observations:

OBC Unit Cleared /Not Cleared

WORK CENTER                                                                         R&QA

# Annexure – 1

- Connector Pin Assignment

I) 6 PIN SERIES-3POWER CHASSIS MOUNTED CONNECTOR DETAILS-M1

CHASSIS MOUNTED- 6 PIN PLUG- D3899920FA35PN (Normal Crimp type)

MATING CONNECTOR- 6 PIN SOCKET -D3899926FA35SN

| PIN NUMBER | SIGNAL NAME | REMARKS |
| --- | --- | --- |
| 1 | 28V-H |  |
| 2 | 28V-L |  |
| 3 | 28V-H |  |
| 4 | 28V-L |  |
| 5 | 28V-H |  |

II) 22 PIN SERIES-3 MIL-1553B PCB MOUNTED CONNECTOR DETAILS-M9

PCB MOUNTED- 22 PIN SOCKET- DTS20F13-35SN-181

MATING CONNECTOR- 22 PIN PLUG -D3899926FC35PN

| PIN NUMBER | SIGNAL NAME |  | PIN NUMBER | SIGNAL NAME |
| --- | --- | --- | --- | --- |
| 1 | BUS1_B0# |  | 15 | SHIELD |
| 2 | BUS 1_B0 |  | 16 | BUS1_A0# |
| 3 | SHIELD |  | 17 | SHIELD |
| 4 | BUS1_A0 |  | 18 | BUS2_B0 |
| 5 | BUS2_A0 |  | 19 | SHIELD |
| 6 | BUS2_A0# |  | 20 | SHIELD |
| 7 | SHIELD |  | 21 | NC |
| 8 | BUS2_B0# |  | 22 | NC |
| 9 | INS_BUS_A0# |  |  |  |
| 10 | INS_BUS_A0 |  |  |  |
| 11 | INS_BUS_B0 |  |  |  |
| 12 | INS_BUS_B0# |  |  |  |
| 13 | NC |  |  |  |
| 14 | NC |  |  |  |

III) 66 PIN SERIES-3 PCB MOUNTED CONNECTOR DETAILS-M7

PCB MOUNTED- 66 PIN SOCKET-DTS20F19-35SN-181

MATING CONNECTOR- 66 PIN PLUG -D3899926FF35PN

| PIN NUMBER | SIGNAL NAME |  | PIN NUMBER | SIGNAL NAME |  | PIN NUMBER | SIGNAL NAME |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | DOP_LOW1 |  | 26 | CDIPH7 |  | 51 | CON_DOPA18 |
| 2 | CON_DOPA6 |  | 27 | CDIPH14 |  | 52 | CON_DOPA17 |
| 3 | CON_DOPA3 |  | 28 | CDIPH15 |  | 53 | INS_RX1+ |
| 4 | CON_DOPA5 |  | 29 | CDIPH23 |  | 54 | INS_RX1- |
| 5 | CON_DOPA4 |  | 30 | OBC_DIP_ LOW2 |  | 55 | F_DOP+ |
| 6 | CON_DOPA1 |  | 31 | CDIPH2 |  | 56 | F_DOP- |
| 7 | CON_DOPA0 |  | 32 | CDIP_SP |  | 57 | CON_DOPA14 |
| 8 | CON_DOPA2 |  | 33 | CDIPH0 |  | 58 | CON_DOPA16 |
| 9 | CON_DOPA8 |  | 34 | IMU_INTR+ |  | 59 | CON_DOPA19 |
| 10 | DOP_ LOW2 |  | 35 | IMU_INTR- |  | 60 | CON_DOPA10 |
| 11 | CON_DOPA7 |  | 36 | CDIPH8 |  | 61 | DIP_LOWSP |
| 12 | CON_DOPA15 |  | 37 | CDIPH9 |  | 62 | CON_DOPA9 |
| 13 | CDIPH22 |  | 38 | CDIPH16 |  | 63 | CON_DOPA12 |
| 14 | CDIPH20 |  | 39 | CDIPH10 |  | 64 | DOP_LOW4 |
| 15 | CDIPH18 |  | 40 | CDIPH3 |  | 65 | CON_DOPA11 |
| 16 | CDIPH17 |  | 41 | CDIPH4 |  | 66 | DOP_ LOW3 |
| 17 | CDIPH12 |  | 42 | OBC_DIP_LOW1 |  |  |  |
| 18 | CDIPH11 |  | 43 | INS_RX3- |  |  |  |
| 19 | CDIPH6 |  | 44 | INS_RX3+ |  |  |  |
| 20 | CDIPH5 |  | 45 | INS_TX3- |  |  |  |
| 21 | CDIPH21 |  | 46 | INS_TX3+ |  |  |  |
| 22 | CDIPH19 |  | 47 | INS_TX1+ |  |  |  |
| 23 | OBC_DIP _LOW3 |  | 48 | INS_TX1- |  |  |  |
| 24 | CDIPH1 |  | 49 | NC |  |  |  |
| 25 | CDIPH13 |  | 50 | CON_DOPA13 |  |  |  |

All these pins are used only for debugging not used at the testing level .

IV) 79 PIN SERIES-3  PCB MOUNTED CONNECTOR DETAILS-M8

PCB MOUNTED- 79 PIN SOCKET- DTS20F21-35SN-181

MATING CONNECTOR- 79 PIN PLUG -D3899926FG35PN

| PIN NUMBER | SIGNAL NAME |  | PIN NUMBER | SIGNAL NAME |  | PIN NUMBER | SIGNAL NAME |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | CON__ANALOG_DIFF_PI |  | 31 | D_GND |  | 61 | CON_SE_CH5 |
| 2 | CON_ANALOG_DIFF_N1 |  | 32 | CON_ANALOG_DIFF_N8 |  | 62 | CON_SE_CH6 |
| 3 | CON_ANALOG_DIFF_N7 |  | 33 | CON_ANALOG_DIFF_P8 |  | 63 | CON_SE_CH7 |
| 4 | CON_ANALOG_DIFF_P7 |  | 34 | CON_ANALOG_DIFF_N4 |  | 64 | CON_SE_CH8 |
| 5 | CON_ANALOG_DIFF_P3 |  | 35 | CON_ANALOG_DIFF_P4 |  | 65 | MIU_DEBUG_TXP |
| 6 | CON_ANALOG_DIFF_N3 |  | 36 | NC |  | 66 | MIU_DEBUG_TXN |
| 7 | CON_ANALOG_DIFF_N5 |  | 37 | TX_GPS1 |  | 67 | A_GND |
| 8 | CON_ANALOG_DIFF_P5 |  | 38 | RX_GPS1 |  | 68 | A_GND |
| 9 | CON_ANALOG_DIFF_P2 |  | 39 | DGPS232 |  | 69 | CON_SE_CH1 |
| 10 | CON_ANALOG_DIFF_N2 |  | 40 | GPS_GND |  | 70 | CON_SE_CH2 |
| 11 | CON_ANALOG_DIFF_P6 |  | 41 | NC |  | 71 | CON_SE_CH3 |
| 12 | CON_ANALOG_DIFF_N6 |  | 42 | INS_ERX_N1 |  | 72 | CON_SE_CH9 |
| 13 | CON_OBC_RX3+ |  | 43 | INS_ERX_P1 |  | 73 | CON_SE_CH10 |
| 14 | CON_OBC_RX3- |  | 44 | INS_ETX_P1 |  | 74 | CON_SE_CH11 |
| 15 | INS_RES- |  | 45 | INS_ETX_N1 |  | 75 | CON_SE_CH12 |
| 16 | INS_RES+ |  | 46 | INS_ETH_GND1 |  | 76 | CON_SE_CH13 |
| 17 | OBC_ERX_P1 |  | 47 | CON_DAC0 |  | 77 | CON_SE_CH14 |
| 18 | OBC_ERX_N1 |  | 48 | CON_DAC1 |  | 78 | CON_SE_CH15 |
| 19 | OBC_ETX_P1 |  | 49 | CON_DAC2 |  | 79 | CON_SE_CH16 |
| 20 | OBC_ETX_N1 |  | 50 | CON_DAC3 |  |  |  |
| 21 | OBC_ETH_GND1 |  | 51 | CON_DAC4 |  |  |  |
| 22 | OBC_RX1+ |  | 52 | CON_DAC5 |  |  |  |
| 23 | OBC_RX1- |  | 53 | CON_DAC6 |  |  |  |
| 24 | OBC_TX1- |  | 54 | CON_DAC7 |  |  |  |
| 25 | OBC_TX1+ |  | 55 | A_GND |  |  |  |
| 26 | OBC_RES- |  | 56 | CON_OBC_CTX3+ |  |  |  |
| 27 | OBC_RES+ |  | 57 | CON_OBC_CTX3- |  |  |  |
| 28 | NC |  | 58 | OBC_TX2+ |  |  |  |
| 29 | OBC_RX2+ |  | 59 | OBC_TX2- |  |  |  |
| 30 | OBC_RX2- |  | 60 | CON_SE_CH4 |  |  |  |

All these pins are used only for debugging not used at the testing level .

# ANNEXURE - 2

## EMI/EMC PROCEDURES

## EMI/EMC TEST PROCEDURES

| SL NO | TEST | TEST DESCRIPTION | APPLICABILITY/LIMITLINE/ TEST PROCEDURE | TEST DURATION | TEST METHOD |
| --- | --- | --- | --- | --- | --- |
| 1 | HESD | Human Electrostatic Discharge Test. | Applicability: Applicable to all interfacing connector is mounted on the equipment and chassis. Test setup and test procedure, As per mil-std-461E | Two pulses shall bed is charged on each of connector and EUT chassis with a time interval of 5 seconds between the pulse. | PREET, INSET, POET |
| 2 | CE102 | Conducted Emission Power leads 10KHz to 10MHz | Applicability : This requirement is applicable from 10KHz to 10MHz for all power leads . | Two pulses shall bed charged on each connector with a time interval of 10 seconds between the pulse. | PREET, INSET, POET |
| 3 | CS101 | Conducted Susceptibility Power range extending 150KHz | Applicability :This is applicable to equipment and subsystem AC & DC input power .This is applicable over the frequency range of 30Hz to 150KHz. | Two pulses shall bed charged on each connector with a time interval of 10 seconds between the pulse. | PREET, INSET, POET |
| 4 | CS114 | Conducted Susceptibility | This requirement is applicable to all interconnecting cables and power cables . | Two pulses shall bed charged on each connector with a time interval of 10 seconds between the pulse. | PREET, INSET, POET |
| 5 | CS115 | Conducted Susceptibility | This requirement is applicable to all aircraft, space and ground system interconnecting cable including power cables. It is also applicable for surface ship and submarine subsystems and equipment when specified by the procuring activity. | Two pulses shall bed charged on each connector with a time interval of 10 seconds between the pulse. | PREET, INSET, POET |
| 6 | CS116 | Conducted Susceptibility | This is applicable to all interconnecting cables including power cables and individual high side power leads . Power returns and neutrals need not be tested individually. | Two pulses shall bed charged on each connector with a time interval of 10 seconds between the pulse. | PREET, INSET, POET |
| 7 | RE102 | Radiated Emission | This test is used to verify that electric field emissions from the EUT and its associated cabling do not exceed specified requirements. The basic intent of the requirement is to protect sensitive receivers from interference coupled through the antennas associated with the receiver. | Two pulses shall bed charged on each connector with a time interval of 10 seconds between the pulse. | PREET, INSET, POET |
| 8 | RS103 | Radiated Susceptibility | This requirement is applicable to equipment and subsystem enclosures and all interconnecting cables. The requirement at the tuned frequency of an antenna – connected receiver is 20Db above the RE102 limit associated with the particular platform application. | Two pulses shall bed charged on each connector with a time interval of 10 seconds between the pulse. | PREET, INSET, POET |

# CE102 Test:-

This test procedure is used to verify that electromagnetic emissions from the EUT(via conducted path)do not exceed the specified requirements for power input leads, including returns. The basic concept is to ensure that the EUT does not corrupt the power quality (allowable voltage distortion) on the power buses present on the platform. This requirement  is applicable from 10KHz to 10MHz for all power leads, including returns that obtain power from other sources not part of the EUT.

# CS101 Test:-

This test procedure is used to verify the ability of the EUT to withstand signals coupled onto power leads. The basic concern is to ensure that equipment performance is not degraded from ripple voltages associated with allowable distortion of power source voltage waveforms. This requirement is applicable to equipment and subsystem AC and DC input power leads, not including returns. If EUT is DC operated ,this requirement is applicable over the frequency range of 30Hz to 150KHz. If the EUT is AC operated, this requirement is applicable starting from the second harmonic of the EUT power frequency and extending to 150KHz.

# CS114 Test:-

This test procedure is used to verify the ability of the EUT to withstand RF signals coupled onto EUT associated cabling. The basic concept is to simulate currents that will be developed on platform cabling from electromagnetic fields generated by antenna transmissions both on and off the platform. This requirement is applicable to all interconnecting cables and power cables.

# CS115 Test:-

This test procedure is used to verify the ability of the EUT to withstand impulse signals coupled on to EUT associated cabling. The basic concern is to protect equipment from fast rise and fall time transients that may be present due to platform switching operations and external transient environments such as lightning and electromagnetic pulse.

# CS116 Test:-

The EUT shall not exhibit any malfunction , degradation of performance or deviation from specified indications, beyond the tolerances indicated in the individual equipment or subsystem specifications, when subjected to a signal having the waveform and having a maximum current as specified. The limit is applicable across the entire specified frequency range. As a minimum, compliance shall be demonstrated at the following frequencies: 0.01, 0.1, 1, 10, 30, and 100 MHz. If there are other frequencies known to be critical to the equipment installation, such as platform resonances, compliances shall also be demonstrated at those frequencies. The test signal repetition rate be no greater than one pulse per second and no less than one pulse every two seconds. The pulses shall be applied for a period of five minutes.

# RE102 Test:-

This test procedure is used to verify that electric field emissions from the EUT and its associated cabling do not exceed specified requirements. The basic intent of the requirement is to protect sensitive receivers from interference coupled through the antennas associated with the receiver.

# RS103 Test:-

This test procedure is used to verify the ability of the EUT and associated cabling to withstand electric fields. The basic concern is to ensure that equipment will operate without degradation in the presence of electromagnetic fields generated by antenna transmissions both onboard and external to the platform.

# Human Electrostatic Discharge Limit (HESD):-

The EUT shall not exhibit any malfunction ,degradation of performance or deviation from specified indications, beyond the tolerance indicated in the individual equipment or subsystems specifications when subjected to a 20KV pulse amplitude (A 150pF capacitor is charged to an amplitude of 20KV and discharged through 150Ω resistor) on each of the connector and EUT chassis with a time interval of 5seconds between the pulses.

NOTE: Note down the PREET, INSET and POET test reading it the functional test leaf for ENTEST given in Appendix B